ARDON®SPARKLELOW S3

SAFETY SHOES

TECHNICAL SHEET

| DIMENSIONS | 37-39 | 40-43 | 44-48 |  |
| --- | --- | --- | --- | --- |
| Carton H x W x D (cm) | 31.5 x 41.5 x 61.5 | 33.5 x 43.5 x 61.5 | 37 x 43.5 x 64 |  |
| Carton (cbm) | 0,0804 | 0,0896 | 0,103 |  |

ARDON®SPARKLELOW S3

SIZE

UNIT

# PACKAGE

Units in a carton

Units in a box

37-48

pair

10 pairs

1 pair

DESCRIPTION

Safety shoes with upper made of high-quality full-grain leather, leather tongue and ankle area, with hydrophobic treatment, with composite toe cap and Kevlar insole. The sole made of PU/rubber material has anti-slip SRC properties, is resistant to contact heat up to 300°C, provides insulation against heat and cold.

| Size EUR | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Insole length mm | - | - | 244 | 250 | 257 | 264 | 270 | 277 | 284 | 290 | 297 | 304 | 310 | 317 | - | - | - |
| Foot length cm | - | - | 23,9 | 24,5 | 25,2 | 25,9 | 26,5 | 27,2 | 27,9 | 28,5 | 29,2 | 29,9 | 30,5 | 31,2 | - | - | - |

Full grain cowhide 1.8 – 2 mm thick, hydrophobic treatment

MESH-type laminated breathable fabric

PU/rubber, direct injection

604 composite

Kevlar

ANATOMIC FOAM – anatomically shaped from EVA foam

MATERIAL

Upper

Lining

Sole

Toe cap

Plate

Insole

G3385

ARTICLE

SAFETY SHOES

TECHNICAL SHEET

TECHNICAL SHEET

# NORMS

# (EU) 2016/425

EN ISO 20345:2011          Personal protective equipment - Safety footwear

Category:  	 S3 HRO SRC HI CI

| Categories | EN ISO 20345 |  |
| --- | --- | --- |
| E - energy absorption in the heel area | ✓ |  |
| FO - resistance to oils and fuels | ✓ |  |
| A - antistatic footwear | ✓ |  |
| SRC - resistance to slipping on ceramic floor tiles with SLS and on steel floors with glycerin | ✓ |  |
| WR - upper resistant to water | ✓ |  |
| Safety toe cap for toe protection | ✓ |  |
| P - resistance to punctures (protective insole) | ✓ |  |
| SRA - slip resistance ceramic floor tiles with SLS |  |  |
| HRO - resistance to contact heatup to 300 °C | ✓ |  |
| insulation of the bottom of the shoe against heat | ✓ |  |
| insulation of the bottom of the shoe against the cold | ✓ |  |
| Splatters of molten metal |  |  |
| ESD – shoe protects against electrostatic discharge and can be used on EPA premises |  |  |
| The shoes do not contain metal parts | ✓ |  |
| - |  |  |
| - |  |  |
| - |  |  |

| CODE | EAN | CARTON WEIGHT (KG) | UNIT WEIGHT (KG) |
| --- | --- | --- | --- |
| G3385/37 | 8592390163004 | 14.00 | 1.400 |
| G3385/38 | 8592390163011 | 15.00 | 1.500 |
| G3385/39 | 8592390163028 | 15.50 | 1.550 |
| G3385/40 | 8592390163035 | 15.80 | 1.580 |
| G3385/41 | 8592390163042 | 16.00 | 1.600 |
| G3385/42 | 8592390163059 | 17.00 | 1.700 |
| G3385/43 | 8592390163066 | 18.00 | 1.800 |
| G3385/44 | 8592390163073 | 18.50 | 1.850 |
| G3385/45 | 8592390163080 | 19.00 | 1.900 |
| G3385/46 | 8592390163097 | 19.50 | 1.950 |
| G3385/47 | 8592390163103 | 20.00 | 2.000 |
| G3385/48 | 8592390163110 | 21.00 | 2.210 |

TECHNICÝ LIST

TECHNICÝ LIST

TECHNICAL SHEET